Card Shuffler: Material List

Everything to buy for the 54-slot wheel shuffler, the poker chip test and the printer. Tick the boxes as you order.

Prices are USD estimates for single quantities, excluding shipping and tax. They come from the project BOM (docs/05-bom.md, electronics/bom.csv) and from typical retail prices; check before buying.

# Summary

| Group | Est. | Note |
| --- | --- | --- |
| Shuffler electronics (E1–E29) | ≈ $98 | Prices from the project BOM |
| Shuffler mechanical (M1–M9) | ≈ $19 | Prices from the project BOM |
| Filament and print consumables | ≈ $72 | + ≈ $20 if you add a dry box |
| Poker chip test parts | ≈ $18 | Only for the chips |
| Tools you don't already own | $0–130 | Skip what you have |
| Printer: P1S refurbished | $339 | Combo only if multi-colour chips |
| Shuffler parts + filament + chip test (no printer, no tools) | ≈ $207 |  |

Already have some of it? A parts drawer with resistors, capacitors, perfboard/wire and M3 screws (E22–E24, E28, M5) saves about $19. Skateboard bearings work for E6. Pen springs (M7) and dead AA cells (M8) are scrap.

# 1. Printer

| ☐ | # | Item | Qty | Spec / what to look for | Est. | Alternatives / where |
| --- | --- | --- | --- | --- | --- | --- |
| ☐ | P1 | Bambu Lab P1S (official refurbished) | 1 | 256 mm bed, enclosed. Check the listing includes spool holder, PSU cable, textured PEI plate, and states the warranty | $339 | Chosen over a $300 eBay unit missing the top glass ($30 to replace, no warranty) |
| ☐ | P1+ | or: P1S Combo (with AMS) | 1 | Only if you will print multi-colour poker chips | check | Worth it at ≈ $100 or less over the standalone; skip for the shuffler alone |

A textured PEI plate comes with the P1S; you do not need the missing top glass for this project.

# 2. Filament and consumables

| ☐ | # | Item | Qty | Spec / what to look for | Est. | Alternatives / where |
| --- | --- | --- | --- | --- | --- | --- |
| ☐ | F1 | PETG filament, 1 kg | 2 | 1.75 mm. Shuffler needs ≈ 0.9 kg; the 2nd spool covers a wheel reprint and the test plates | ≈ $40 | Bambu PETG HF, Polymaker, eSun, Overture |
| ☐ | F2 | PLA filament, 1 kg | 1 | 1.75 mm. ≈ 0.35 kg for rollers, idlers, lid, base | ≈ $18 | Any brand |
| ☐ | F3 | Glue stick | 1 | PETG release on the textured PEI plate | ≈ $3 | Bambu glue stick or any PVP stick |
| ☐ | F4 | Hot glue sticks (+ gun if you have none) | 1 | Fixes beam LEDs, phototransistors, N20 motors, leads | ≈ $5 | Used in assembly steps; not in the main BOM |
| ☐ | F5 | Electrical / masking tape | 1 | One wrap on the M8 rod at each bearing seat; ballast cells | ≈ $3 | Not in the main BOM |
| ☐ | F6 | 600-grit sandpaper | 1 | Gate lip, shutter face, wheel fins | ≈ $3 | Not in the main BOM |
| ☐ | F7 | Filament dry box (optional) | 1 | PETG absorbs moisture; skip if you buy the AMS combo | ≈ $20 | Sealed tub + desiccant works too |

# 3. Electronics

| ☐ | # | Item | Qty | Spec / what to look for | Est. | Alternatives / where |
| --- | --- | --- | --- | --- | --- | --- |
| ☐ | E1 | ESP32 DevKitC V4 (WROOM-32 30-pin) | 1 | ESP32-WROOM-32, CP2102/CH340 USB, 30-pin | $6 | Alt: ESP32-S3 DevKitC (pin remap). EBay ~$5, LCSC $8.7, Mouser $10 |
| ☐ | E2 | TMC2208 stepper driver stepstick | 1 | standalone mode, with heatsink | $5 | Alt: A4988 or DRV8825 (3S pack only). UK £3.25-4.45, US $6-20 |
| ☐ | E3 | NEMA 17 stepper motor | 1 | 42 mm, 1.5-1.7 A, 4-wire, 5 mm shaft (17HS4401 class) | $10 | Alt: 17HS4023 (1 A) with Vref 0.7 V. StepperOnline, Amazon, AliExpress |
| ☐ | E4 | Shaft coupler 5 mm to 8 mm | 1 | flexible aluminium 19 x 25 mm | $2.50 | Alt: rigid coupler. Ender-3 Z-axis kits include one |
| ☐ | E5 | M8 threaded rod 200 mm + 4 nyloc nuts + 6 washers | 1 | zinc plated, cut to 165 mm | $2 | Alt: 8 mm smooth rod with a filed flat and set screws. Hardware store |
| ☐ | E6 | 608ZZ bearing | 2 | 8 x 22 x 7 mm | $1.50 | Alt: 608-2RS. Skate bearings, Amazon 10-pack ~$7 |
| ☐ | E7 | MG90S micro servo | 1 | metal gear, 180 deg, with horns | $3 | Alt: SG90. Waveshare $2.59, eBay 4-pack $13.95 |
| ☐ | E8 | N20 micro gearmotor 6 V 300 rpm | 3 | N20 metal gearbox, 3 mm D-shaft | $12 | Alt: N20 200-500 rpm. Robu.in ~$3, Amazon 2-packs ~$9 |
| ☐ | E9 | DRV8833 dual H-bridge module | 2 | 2.7-10.8 V, 1.5 A/ch | $6 | Alt: TB6612FNG module. Pololu $4.95, generic $2-3 |
| ☐ | E10 | 3S Li-ion battery pack with BMS | 1 | 11.1 V 2200-2600 mAh 18650 3S1P, BMS >= 5 A, XT30 or JST-XH | $18 | Alt: any protected 3S pack <= 75 x 60 x 22 mm. Amazon, robotics suppliers |
| ☐ | E11 | 12.6 V 1 A Li-ion charger | 1 | CC/CV, 5.5x2.1 barrel | $6 | Alt: 12.6 V 1.5-2 A. EBay £3-4 to $19 |
| ☐ | E12 | DC barrel jack panel mount 5.5x2.1 | 1 | with nut | $1 |  |
| ☐ | E13 | Buck converter 5 V 3 A | 1 | MP1584EN or LM2596 adjustable | $2 | Alt: fixed 5 V 3 A module |
| ☐ | E14 | Toggle switch SPST | 1 | >= 3 A DC, 6 mm bushing | $1.50 | Alt: rocker |
| ☐ | E15 | Push button momentary 12 mm | 1 | panel mount | $2 | Alt: arcade button |
| ☐ | E16 | WS2812B pixel breakout | 1 | single pixel | $1 | Alt: 3 LEDs |
| ☐ | E17 | Passive piezo buzzer | 1 | 5 V passive | $1 | Alt: active buzzer |
| ☐ | E18 | IR LED 940 nm 3 mm | 5 | TSAL4400 / IR204 | $1.75 | Alt: 5 mm (enlarge holes). DigiKey/RS $0.25-0.45 |
| ☐ | E19 | IR phototransistor 3 mm | 5 | TEFT4300 / PT204-6B | $2 | Alt: 5 mm PT334. DigiKey/RS ~$0.30 |
| ☐ | E21 | NPN transistor 2N2222 / BC547 | 1 | TO-92 | $0.20 | Alt: logic-level N-MOSFET |
| ☐ | E22 | Resistor set | 1 | 150 x2, 68 x2, 330 x2, 470, 10 k x5, 100 k, 33 k, 100 ohm | $1 |  |
| ☐ | E23 | Capacitors | 1 | 100 uF 25 V x3, 470 uF 10 V, 100 nF x3 | $1 |  |
| ☐ | E24 | Perfboard 70x50 + headers + JST-XH kit + wire | 1 | female headers, JST-XH 2.54 crimp kit, 22-24 AWG | $8 | Alt: Dupont |
| ☐ | E25 | Polyfuse 3 A | 1 | resettable, or blade fuse holder + 3 A | $1 |  |
| ☐ | E26 | MT3608 boost module (noise board) | 1 | 2-24 V in, adjustable, set to 15.0 V | $1 | Alt: any small adjustable boost to 12-18 V |
| ☐ | E27 | 2N3904 NPN transistor (noise board) | 2 | TO-92; Q1 avalanche junction, Q2 amplifier | $0.20 | Alt: BC547 (check its E-B breakdown) |
| ☐ | E28 | Noise board passives | 1 | 22 k, 4.7 k, 470 k, 1 k, 100 ohm; 100 nF; 10 uF 25 V x2 | $0.50 |  |
| ☐ | E29 | Perfboard 60 x 40 mm (noise board) | 1 | 2.54 mm pitch, corner holes for M2 (56 x 36 mm pitch) | $1 | Alt: cut from a larger board |
|  |  | Electronics subtotal |  |  | ≈ $98 |  |

## Buying tips

- "Ender-3 Z-axis kit" listings include the 5 → 8 mm coupler (E4).

- N20 motors (E8): buy three identical ones, 3 mm D-shaft, 300 rpm.

- Noise board (E26–E29): the avalanche random source. Optional; without it set USE_AVALANCHE 0 in the firmware.

- Battery (E10): it must fit in 75 × 60 × 22 mm and have a BMS rated for at least 5 A.

- Cheapest build: A4988 instead of TMC2208, SG90 instead of MG90S, 17HS4023 motor, skate bearings, no noise board: about $88 total parts.

# 4. Mechanical

| ☐ | # | Item | Qty | Spec / what to look for | Est. | Alternatives / where |
| --- | --- | --- | --- | --- | --- | --- |
| ☐ | M1 | O-rings ID 20 x CS 3.5 mm nitrile | 5 | feed roller (4 + spare) | $2 | Alt: silicone. Metric O-ring assortment ~$8 covers M1-M3 |
| ☐ | M2 | O-rings ID 10 x CS 3.0 mm nitrile | 9 | two nip rollers (8 + spare) | $2.25 | Assortment |
| ☐ | M3 | O-rings 9 x 2 mm | 4 | idler wheels | $0.80 | Alt: plain printed wheels. Assortment |
| ☐ | M4 | Self-adhesive felt sheet 1.5 mm | 1 | chute end wall, lid pad, optional shroud lining | $3 | Alt: EVA foam |
| ☐ | M5 | M3 screw assortment | 1 | M3 x 6/8/10/12/16/20 (~80 pcs), M3 x 40 x4, M3 x 50 x2, M3 x 60 x1, nuts, 3 grub | $8 | Alt: heat-set inserts optional |
| ☐ | M6 | M2 screws | 9 | M2 x 8 x4 servo flange, M2 x 8 x4 noise board, M2 x 12 x1 horn pin | $0.90 |  |
| ☐ | M7 | Compression springs from ballpoint pens | 4 | ~5 mm OD, 15 mm | $0 | Alt: rubber bands. Scrap |
| ☐ | M8 | AA cells (dead) for lid ballast | 2 | any | $0 | Alt: coins/washers. Scrap |
| ☐ | M9 | Velcro strap + foam pad | 1 | battery retention | $2 | Alt: zip ties |
|  |  | Mechanical subtotal |  |  | ≈ $19 |  |

One metric O-ring assortment (about $8) covers M1–M3. The M3 assortment must include the long screws: 4 × 40 mm, 2 × 50 mm, 1 × 60 mm, plus nyloc nuts and three M3 × 5 grub screws.

# 5. Poker chip test

For chips/test_plate.stl: three chips plus the bearing fit test, one pause at 4.0 mm.

| ☐ | # | Item | Qty | Spec / what to look for | Est. | Alternatives / where |
| --- | --- | --- | --- | --- | --- | --- |
| ☐ | C1 | M8 fender washer, DIN 9021 | 10+ | Ø24 OD × Ø8.4 ID × 2.0 mm, ≈ 6 g each. Measure: the pocket is sized for exactly this | ≈ $6 | Hardware store or a 25-pack online |
| ☐ | C2 | NFC tag, NTAG215, Ø25 mm, ON-METAL / anti-metal | 10 | Ferrite-backed, ≈ 1 mm thick. A normal sticker tag will NOT read on the washer | ≈ $12 | Search "anti-metal NFC NTAG215 25mm" |

# 6. Tools

Skip anything you already own.

| ☐ | # | Item | Qty | Spec / what to look for | Est. | Alternatives / where |
| --- | --- | --- | --- | --- | --- | --- |
| ☐ | T1 | Digital calipers | 1 | Measure washers, tags, test prints | ≈ $15 | Required |
| ☐ | T2 | Feeler gauge set | 1 | Includes a 1.5 mm blade for the wheel slots | ≈ $8 | Required |
| ☐ | T3 | Hex + Phillips drivers (M3, M2) | 1 | 2.5 mm hex for M3, 1.5 mm for M3 grubs | ≈ $10 | Required |
| ☐ | T4 | Pliers, flush/side cutters | 1 | Cutters also remove the feeder_deck print ribs | ≈ $10 | Required |
| ☐ | T5 | Soldering iron + solder | 1 | Temperature controlled | ≈ $25 | Required |
| ☐ | T6 | JST-XH crimper | 1 | Or buy pre-crimped JST-XH leads | ≈ $20 | Required (or pre-crimped) |
| ☐ | T7 | Needle files | 1 | Gate window, edges | ≈ $8 | Required |
| ☐ | T8 | Hacksaw | 1 | Cut the M8 rod to 165 mm | ≈ $10 | Required |
| ☐ | T9 | Multimeter | 1 | Set the TMC2208 Vref and the buck output to 5 V | ≈ $15 | Required |
| ☐ | T10 | USB cable for the ESP32 | 1 | Micro-USB or USB-C to match your DevKitC; data, not charge-only | ≈ $5 | Required |
| ☐ | T11 | M3 tap, 8 mm drill, heat-set insert tip | — | Only if holes or the wheel bore print tight | — | Optional |

# Optional

| ☐ | # | Item | Qty | Spec / what to look for | Est. | Alternatives / where |
| --- | --- | --- | --- | --- | --- | --- |
| ☐ | O1 | TCRT5000 module (optional deck-loaded sensor) | 1 |  | $1 |  |